1-12-24.  Custom AIR-DNA-FFPE TCR repertoire profiling assay, Rusul Almarayaty (NWU).

11 Samples – flow cells (#341) > 300-n paired-end read (medium-throughput)> NextSeq500

Sample Description > please find in the attached Excel File

Experiment description:

QC of provided RNA samples.  Customer provided 11 DNA FFPE samples (range 1.5-8 ug)purified from human sarcoma tumors. Additional measurement confirms the provided concentration of DNA samples.  Considering low content of lymphocyte cells and differences in DNA degradation level by FFPE treatment, whole amount of provided DNA (1.5-8 ug)was used in AIR-DNA TCR assay as a starting material for all samples (1-11).

QC of amplified TCR library products.

Analysis of amplified TCR products (after 2nd amplification step) show significant differences in the yield of amplified TCR CDR3 products, no significant primer dimers or off-target products.  The yield of amplified cDNA products:

7,9,10: 100%, 8,11: 30-40%, 6: 15-20%, 3: 5%, 1,4: 0.5%, 2,5: <0.1% (practically undetectable considering that amount of DNA in these two samples was appr. 7-8 ug).  See Gel Electrophoresis and Fragment Analyzer attached image file with QC of PCR products.

Protocol AIR-RNA assay:

Step 1. RevGSP extension.  DNA (1.5-8ug) (in 120 ul) was extended by adding 20 ul of master mix with pool of Reverse AIR TCR GSPs, incubated at 98C, 1min, 68C, 10 min and treated with 2 ul of primer removal enzyme mix, at 37C, 20min, 95C,5min.

Step 2.  Fwd GSP extension. Rev GSP-extended DNA product (in 120 ul) was extended by adding 6 ul of pool of Forward AIR TCR GSPs, incubated at 98C, 1min, 68C, 10 min and treated with 2 ul of primer removal enzyme mix, at 37C, 20min, 95C,5min.

Step 4-1st PCR.  FwdGSP-extended DNA was diluted in PCR master mix (100ul), and anchored cDNA fragments were amplified in 2x110-ul of Multiplex DNA polymerase reaction mix with universal anchor PCR primers for 24 cycles: (98C, 20sec, 65C, 10sec, 72C,30sec).

Step 5-2nd PCR.  2ul aliquotes of 1st PCR were added in 50ul of master mix for 2nd PCR step (each tube has unique Dual Index primers) and amplified using unique combination of Fwd-P5-Index and Rev-P7-Index for 8-11 cycles (15 cycles for samples 2,5; 15 cycles for 1,4; 13 cycles for 3 and 6; 11 cycles for 11, 10 cycles for 8 and 9 cycles for 7,9,10), treated with primer removal enzyme mix (1-ul) at 37C for 30min. PCR products were analyzed in Agarose gel and Fragment analyzer (see attached file), combined at equal amount if possible (for samples for samples 2,5 whole volume of 2nd PCR reaction) based on Fragment analyzer smear analysis data (180-450bp range), and purified using AMPpure magnetic beads (0.6X-1.5X volume). The purified cDNA products were quantitated by Qubit fluorescence measurement, and diluted to 10 nM (1.6 ng/ul) for next-generation sequencing using NextSeq500.

Program:

Read1:eSeqDNA-Fwd>148c; Ind1:eSeqIND-Fwd>10c; Ind2:eSeqIND-Rev>10c;Read2: eSeqDNA-Rev>148c.

DriverMap AIR assay Amplicon Structure

eSeqDNA-Fwd

FP5              UDPIndex10 AGCAGCAGCACCGACCAGCAGACA  F          ACGGCGACCACCGAGATCTACACNNNNNNNNNNAGCAGCAGCACCGACCAGCAGACA-GSP-DNA-

TGCCGCTGGTGGCTCTAGATGTGNNNNNNNNNNTCGTCGTCGTGGCTGGTCGTCTGT-GSP-DNA-

TCGTCGTCGTGGCTGGTCGTCTGT

eSeqIND-Rev

eSeqIND-Fwd

UMI14       TCTGTGCTGGTCGGTGCTCGTCGT

-DNA-GSP-NNNNNNNNNNNNNN-TCTGTGCTGGTCGGTGCTCGTCGTNNNNNNNNNNTATCTCGTATGCCGTCTTCTGCT

-DNA-GSP-NNNNNNNNNNNNNN-AGACACGACCAGCCACGAGCAGCANNNNNNNNNNATAGAGCATACGGCAGAAGACGA

R                 AGACACGACCAGCCACGAGCAGCA  UDPIndex10        RP7

eSeqDNA-Rev