12-22-23.  Custom AIR-RNA TCR repertoire profiling assay, Rusul Almarayaty (NWU).

11 Samples – flow cells (#340) > 300-n paired-end read (high-throughput)> NextSeq500

Sample Description > please find in the attached Excel File

Experiment description:

QC of provided RNA samples.  Customer provided 11 RNA FFPE samples purified from human sarcoma tumors. Additional measurement confirm the provided concentration of RNA samples.  Considering low content of lymphocyte cells and differences in RNA degradation level by FFPE treatment, 1 ug of RNA was used in AIR RNA TCR assay as a starting material for all samples (1-11).

QC of amplified TCR library products.

Analysis of amplified TCR cDNA products (after 2nd amplification step) show significant differences in the yield of amplified TCR CDR3 products, no primer dimers or off-target products.  The yield of amplified cDNA products:

9,10: 100%, 7,8: 5%, 1: 1%, 4,6,11: 0.2%, 2,3,5: <0.1%.  See Fragment Analyzer attached image file with QC of PCR products.

Protocol AIR-RNA assay:

Step 1- RevGSP binding to mRNA.  Total RNA (1,000 ng of each sample)was incubated with mix of Reverse mouse AIR TCR Gene-specific (GS) primers in 20ul of 1xHybridization buffer at 70C, 5 min, 60C for 60 min, cool down to 25C. Hybridized RNA-RevGSP products were purified with 1.4x volume (28 ul) of SPRIselect beads, beads were washed 2x with 80% ethanol.

Step 2- Rev GSP extension > cDNA synthesis. The washed magnetic beads were resuspended in 23 ul of 1xRT-extension master mix, and 20.5 ul of master mix solution without beads was incubated at 50C for 30 min, and 72C for 10min.

Step 3.  Fwd GSP extension. cDNA product (in 20 ul) was extended by adding 20 ul of master mix with pool of Forward AIR TCR GSPs, incubated at 98C, 1min, 68C, 10 min and treated with 2 ul of primer removal enzyme mix, at 37C, 30min, 95C,5min.

Step 4-1st PCR.  FwdGSP-extended cDNA was diluted in PCR master mix (60ul), and anchored cDNA fragments were amplified in 100-ul of Multiplex DNA polymerase reaction mix with universal anchor PCR primers for 21 cycles: (98C, 20sec, 65C, 10sec, 72C,30sec).

Step 5-2nd PCR.  2ul aliquotes of 1st PCR were added in 50ul of master mix for 2nd PCR step (each tube has unique Dual Index primers) and amplified using unique combination of Fwd-P5-Index and Rev-P7-Index for 8-11 cycles (11 cycles for samples 1-6 and 11), treated with primer removal enzyme mix (1-ul) at 37C for 30min. PCR products were analyzed in Fragment analyzer (see attached file), combined at equal amount if possible (for samples for samples 2,3,4,5 whole volume of 2nd PCR reaction) based on Fragment analyzer smear analysis data (250-450bp range), and purified using AMPpure magnetic beads (1.5X volume). The purified cDNA products were quantitated by Qubit fluorescence measurement, and diluted to 10 nM (1.6 ng/ul) for next-generation sequencing using NextSeq500.

Program:

Read1:eSeqDNA-Fwd>148c; Ind1:eSeqIND-Fwd>10c; Ind2:eSeqIND-Rev>10c;Read2: eSeqDNA-Rev>148c.

DriverMap AIR assay Amplicon Structure

eSeqDNA-Fwd

FP5              UDPIndex10 AGCAGCAGCACCGACCAGCAGACA  F          ACGGCGACCACCGAGATCTACACNNNNNNNNNNAGCAGCAGCACCGACCAGCAGACA-GSP-DNA-

TGCCGCTGGTGGCTCTAGATGTGNNNNNNNNNNTCGTCGTCGTGGCTGGTCGTCTGT-GSP-DNA-

TCGTCGTCGTGGCTGGTCGTCTGT

eSeqIND-Rev

eSeqIND-Fwd

UMI14       TCTGTGCTGGTCGGTGCTCGTCGT

-DNA-GSP-NNNNNNNNNNNNNN-TCTGTGCTGGTCGGTGCTCGTCGTNNNNNNNNNNTATCTCGTATGCCGTCTTCTGCT

-DNA-GSP-NNNNNNNNNNNNNN-AGACACGACCAGCCACGAGCAGCANNNNNNNNNNATAGAGCATACGGCAGAAGACGA

R                 AGACACGACCAGCCACGAGCAGCA  UDPIndex10        RP7

eSeqDNA-Rev